# 状态

### 碰撞检测状态

- 上位机查询碰撞检测状态

  - 命令字：0x3200

    - "clear":0

      - 是否清除理论扭矩和实际扭矩差别；int类型，范围[0,1]

{

"clear":0

}

- 控制器回复上位机碰撞检测状态

  - 命令字：0x3201

    - "Difference_torq":[0.0,0.0,0.0,0.0,0.0,0.0,0.0]

      - 理论与反馈差值列表；double类型

      - 长度为7

    - "actual_torq":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

      - 扭矩反馈值列表；double类型

      - 长度为7

    - "detect_torq":[50.0,50.0,50.0,50.0,50.0,50.0,0.0],

      - 碰撞检测阈值列表；double类型

      - 长度为7

    - "max_torq":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

      - 插值峰值列表；double类型

      - 长度为7

    - "target_torq":[0.0,0.0,0.0,0.0,0.0,0.0,0.0]

      - 扭矩理论值列表；double类型

      - 长度为7

{

"Difference_torq":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

"actual_torq":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

"detect_torq":[50.0,50.0,50.0,50.0,50.0,50.0,0.0],

"max_torq":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

"target_torq":[0.0,0.0,0.0,0.0,0.0,0.0,0.0]

}

### 指令运行百分比

- 上位机查询运动指令已完成百分比

  - 命令字：0x3202

    - data：无

- 上控制器回复运动指令已完成百分比

  - 命令字：0x3203

    - "completionRatio":0.11

      - 运行百分比；double类型；取值范围[0,1]

      - 最大值为1

{

"completionRatio"：0.11

}

### 当前位置

- 上位机查询机器人坐标位置

  - 命令字：0x3210 0x3211

    - "coord":0

      - 查询当前坐标的位置；int类型，范围[0,4]

      - 0表示关节坐标，1表示直角坐标，2表示工具坐标，3表示用户坐标，4表示电机坐标

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"coord":0,

"robot":1

}

- 控制器回复上位机机器人坐标位置(0x3210)

  - 命令字：0x3212

    - "configuration":8

      - 当前位置形态值；int类型，范围[0,8]

    - "coord":0

      - 当前位置坐标类型；int类型，范围[0,4]

      - 0表示关节坐标，1表示直角坐标，2表示工具坐标，3表示用户坐标，4表示电机坐标

    - "pos":[0.8047034822,0.0,0.0,0.033915260824,0.0,-0.008860939141,0.0]

      - 获取到的弧度制当前位置信息；double类型

    - "posDeg":[0.8047034822,0.0,0.0,0.033915260824,0.0,-0.008860939141,0.0]

      - 获取到的角度制当前位置信息；double类型

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"configuration":8,

"coord":0,

"pos":[0.8047034822,0.0,0.0,0.033915260824,0.0,-0.008860939141,0.0],

"posDeg":[0.8047034822,0.0,0.0,0.033915260824,0.0,-0.008860939141,0.0],

"robot":1

}

- 控制器回复上位机机器人坐标位置(0x3211)

  - 命令字：0x3213

    - "coord":0

      - 当前位置坐标类型；int类型，范围[0,4]

      - 0表示关节坐标，1表示直角坐标，2表示工具坐标，3表示用户坐标，4表示电机坐标

    - "isDeg":0

      - 是否为角度制；int类型，范围[0,1]

    - "position":[0.8047034822,0.0,0.0,0.033915260824,0.0,-0.008860939141,0.0]

      - 获取到的弧度制当前位置信息；double类型

    - "positionDeg":[0.8047034822,0.0,0.0,0.033915260824,0.0,-0.008860939141,0.0]

      - 获取到的角度制当前位置信息；double类型

    - "posture":8

      - 当前位置形态值；int类型，范围[0,8]

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"coord":0,

"isDeg":0,

"position":[0.8047034822,0.0,0.0,0.033915260824,0.0,-0.008860939141,0.0],

"positionDeg":[0.8047034822,0.0,0.0,0.033915260824,0.0,-0.008860939141,0.0],

"posture":8,

"robot":1

}

- 上位机查询双机协作下机器人坐标位置

  - 命令字：0x3214

    - "coord1":0

      - 获取机器人1位置的坐标类型；int类型，范围[0,3]

      - 0表示关节坐标，1表示直角坐标，2表示工具坐标，3表示用户坐标

    - "coord2":0

      - 获取机器人2位置的坐标类型；int类型，范围[0,3]

      - 0表示关节坐标，1表示直角坐标，2表示工具坐标，3表示用户坐标

    - "robot": 1

      - 双机协作第一个机器人号；int类型，范围[1,4]

      - 默认为1

    - "robot": 2

      - 双机协作第二个机器人号；int类型，范围[1,4]

      - 默认为2

{

"coord1": 1,

"coord2": 1,

"robot1": 1,

"robot2": 2

}

- 控制器回复上位机双机协作下机器人坐标位置

  - 命令字：0x3215

    - "coord1":0

      - 机器人1位置的坐标类型；int类型，范围[0,3]

      - 0表示关节坐标，1表示直角坐标，2表示工具坐标，3表示用户坐标

    - "coord2":0

      - 机器人2位置的坐标类型；int类型，范围[0,3]

      - 0表示关节坐标，1表示直角坐标，2表示工具坐标，3表示用户坐标

    - "pos1":[-0.0,0.0,3.077931758984,-0.0,0.0,-0.0,0.0]

      - 机器人1弧度制下的坐标；double类型

    - "pos1Deg":[-0.0,0.0,3.077931758984,-0.0,0.0,-0.0,0.0],

      - 机器人1角度制下的坐标；double类型

    - "pos2":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

      - 机器人2弧度制下的坐标；double类型

    - "pos2Deg":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

      - 机器人2角度制下的坐标；double类型

    - "robot": 1

      - 双机协作第一个机器人号；int类型，范围[1,4]

      - 默认为1

    - "robot": 2

      - 双机协作第二个机器人号；int类型，范围[1,4]

      - 默认为2

{

"coord1":0,

"coord2":0,

"pos1":[-0.0,0.0,3.077931758984,-0.0,0.0,-0.0,0.0],

"pos1Deg":[-0.0,0.0,3.077931758984,-0.0,0.0,-0.0,0.0],

"pos2":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

"pos2Deg":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

"robot1":1,

"robot2":2

}

### 电机负载值

- 说明

- 上位机查询机器人电机扭矩过载值

  - 命令字：0x3220

    - "robot_num":1

      - 查询机器人号；int类型，范围[1,4]

{

"robot_num":1

}

- 控制器回复上位机机器人电机扭矩过载值

  - 命令字：0x3221

    - "torq_limit_enable":false

      - 电机过载保护使能；bool类型

    - "torq_limit_min_ensure_time":10ms

      - 电机过载保护最小确认时间；double类型，范围[10,9999]

    - "torq_limit_robot":[800.0,1400.0,1000.0,1200.0,800.0,700.0]

      - 电机扭矩过载值列表；double类型，单位‰

      - 长度为当前机器人的轴数量

    - "torq_limit_sync":[-0.0,-0.0,0.0,0.0,-0.0]

      - 外部轴电机扭矩过载值列表；double类型，单位‰

      - 长度为当前机器人绑定的外部轴数量

{

"torq_limit_enable":false,

"torq_limit_min_ensure_time":10,

"torq_limit_robot":[800.0,1400.0,1000.0,1200.0,800.0,700.0],

"torq_limit_sync":[-0.0,-0.0,0.0,0.0,-0.0]

}

### 轴速度

- 说明

- 上位机查询机器人轴速度

  - 命令字：0x3230

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"robot":1

}

- 控制器回复上位机机器人轴速度

  - 命令字：0x3231

    - "actualLineVel":0.0

      - 机器人末端当前线速度；double类型

    - "axisActualVel":[0.0,0.0,0.0,0.0,0.0,0.0,0.0]

      - 机器人各轴当前速度列表；double类型，单位°/s

      - 长度为7

    - "axisActualVelSync":[0.0,0.0,0.0,0.0,0.0]

      - 机器人外部轴各轴当前速度列表；double类型，单位°/s

      - 长度为5

    - "maxActualLineVel":0.0,

      - 机器人末端最大线速度；double类型，单位mm/s

    - "maxAxisActualVel":[0.0,0.0,0.0,0.0,0.0,0.0,0.0]

      - 机器人各轴最大速度列表；double类型，单位°/s

      - 长度为7

    - "maxAxisActualVelSync":[0.0,0.0,0.0,0.0,0.0],

      - 机器人外部轴各轴最大速度列表；double类型，单位°/s

      - 长度为5

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"actualLineVel":0.0,

"axisActualVel":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

"axisActualVelSync":[0.0,0.0,0.0,0.0,0.0],

"maxActualLineVel":0.0,

"maxAxisActualVel":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

"maxAxisActualVelSync":[0.0,0.0,0.0,0.0,0.0],

"robot":1

}

### 福德伺服温度

- 说明

  - 福德伺服专属功能

- 上位机查询福德伺服温度是否存在

  - 命令字：0x3250

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"robot":1

}

- 控制器回复上位机福德伺服温度是否存在

  - 命令字：0x3251

    - "IsExist":false

      - 福德伺服温度是否存在；bool类型

{

"IsExist":false

}

- 控制器向示教器发送延时时间响应

  - 命令字：0x3252

    - "delaytime":4

      - 延迟时间；double类型，单位s

    - "robot": 1

      - 当前机器人号；int类型，范围[1,4]

{

"delaytime":4,

"robot":1

}

### 电机状态

- 说明

  - 电机参数查询，伺服温度仅福德伺服温度存在时可用

- 上位机查询机器人电机转速

  - 命令字：0x3260

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"robot":1

}

- 控制器回复上位机机器人电机转速

  - 命令字：0x3261

    - "maxVel":[0,0,0,0,0,0,0]

      - 机器人最大电机转速列表；double类型，单位rpm

      - 长度为7

    - "maxVelSync":[0,0,0,0,0]

      - 机器人外部轴最大电机转速列表；double类型，单位rpm

      - 长度为5

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

    - "vel":[0,0,0,0,0,0,0],

      - 机器人当前电机转速列表；double类型，单位rpm

      - 长度为7

    - "velSync":[0,0,0,0,0]

      - 机器人外部轴当前电机转速列表；double类型，单位rpm

      - 长度为5

    - "slave_vel":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人主轴及从动轴电机速度列表；double类型；单位rpm

      - 存在7个元素；为机器人的7个关节

      - 元素存在4个元素，第1位代表主轴，2-4位代表从动轴

    - "slave_max_vel":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人主轴及从动轴最大电机速度列表；double类型；单位rpm

      - 存在7个元素；为机器人的7个关节

      - 元素存在4个元素，第1位代表主轴，2-4位代表从动轴

    - "slave_vel_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人外部轴主轴及从动轴电机速度列表；double类型；单位rpm

      - 存在5个元素；为机器人的5个关节

      - 元素存在4个元素，第1位代表主轴，2-4位代表从动轴

    - "slave_max_vel_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人外部轴主轴及从动轴电机速度列表；double类型；单位rpm

      - 存在5个元素；为机器人的5个关节

      - 元素存在4个元素，第1位代表主轴，2-4位代表从动轴

{

"maxVel":[0,0,0,0,0,0,0],

"maxVelSync":[0,0,0,0,0],

"robot":1,

"vel":[0,0,0,0,0,0,0],

"velSync":[0,0,0,0,0],

"slave_vel":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_max_vel":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_vel_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_max_vel_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

}

- 上位机查询机器人电机扭矩

  - 命令字：0x3262

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"robot":1

}

- 控制器回复上位机机器人电机扭矩

  - 命令字：0x3263

    - "maxTheoTorq":[0,0,0,0,0,0,0],

      - 机器人最大电机理论扭矩列表；double类型，单位‰

      - 长度为7

    - "maxTorq":[0,0,0,0,0,0,0],

      - 机器人最大电机扭矩列表；double类型，单位‰

      - 长度为7

    - "maxVelSync":[0,0,0,0,0]

      - 机器人外部轴最大电机扭矩列表；double类型，单位‰

      - 长度为5

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

    - "theoTorq":[0,0,0,0,0,0,0],

      - 机器人当前电机理论扭矩列表；double类型，单位‰

      - 长度为7

    - "torq":[0,0,0,0,0,0,0],

      - 机器人当前电机扭矩列表；double类型，单位‰

      - 长度为7

    - "torqSync":[0,0,0,0,0]

      - 机器人外部轴当前电机扭矩列表；double类型，单位‰

      - 长度为5

    - "slave_torq":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人主轴及从动轴电机扭矩列表；double类型，单位‰

      - 存在7个元素，为7个关节

      - 元素的元素存在4个，第1位：主轴，2-4：从动轴

    - "slave_max_torq":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]

      - 机器人主轴及从动轴电机最大扭矩列表；double类型，单位‰

      - 存在7个元素，为7个关节

      - 元素的元素存在4个，第1位：主轴，2-4：从动轴

    - "slave_torq_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人外部轴主轴及从动轴电机扭矩列表；double类型，单位‰

      - 存在7个元素，为7个关节

      - 元素的元素存在4个，第1位：主轴，2-4：从动轴

    - "slave_max_torq_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人外部轴主轴及从动轴最大电机扭矩列表；double类型，单位‰

      - 存在5个元素，为五个外部轴

      - 元素的元素存在4个，第1位：主轴，2-4：从动轴

{

"maxTheoTorq":[0,0,0,0,0,0,0],

"maxTorq":[0,0,0,0,0,0,0],

"maxTorqSync":[0,0,0,0,0],

"robot":1,

"theoTorq":[0,0,0,0,0,0,0],

"torq":[0,0,0,0,0,0,0],

"torqSync":[0,0,0,0,0],

"slave_torq":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_max_torq":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],

"slave_torq_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_max_torq_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

}

- 上位机查询机器人电机负载率

  - 命令字：0x3264

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"robot":1

}

- 控制器回复上位机机器人电机负载率

  - 命令字：0x3265

    - "externalOverload":[0.0,0.0,0.0,0.0,0.0],

      - 机器人外部轴当前电机负载列表；double类型，单位‰

      - 长度为5

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

    - "robotOverload":[0.0,0.0,0.0,0.0,0.0,0.0,0.0]

      - 机器人当前电机负载列表；double类型，单位‰

      - 长度为7

    - "slave_motor_rbt_over_load":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人主轴及从动轴电机负载列表；double类型，单位‰

      - 存在7个元素，机器人的七个关节

      - 元素存在4个元素，第1位：主轴，2-3位：从动轴

    - "slave_motor_sync_over_load":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人外部轴主轴及从动轴电机负载列表；double类型，单位‰

      - 存在5个元素，机器人的五个外部轴

      - 元素存在4个元素，第1位：主轴，2-3位：从动轴

{

"externalOverload":[0.0,0.0,0.0,0.0,0.0],

"robot":1,

"robotOverload":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

"slave_motor_rbt_over_load":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_motor_sync_over_load":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

}

- 上位机查询机器人编码器位置

  - 命令字：0x3266

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"robot":1

}

- 控制器回复上位机机器人编码器位置

  - 命令字：0x3267

    - "reality":[0,0,0,0,0,0,0],

      - 机器人编码器位置实际值列表；double类型

      - 长度为7

    - "maxVelSync":[0,0,0,0,0]

      - 机器人外部轴编码器位置实际值列表；double类型

      - 长度为5

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

    - "target":[0,0,0,0,0,0,0],

      - 机器人编码器位置目标值列表；double类型

      - 长度为7

    - "targetSync":[0,0,0,0,0]

      - 机器人外部轴编码器位置目标值列表；double类型

      - 长度为5

    - "slave_reality":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人主轴及从动轴编码器位置实际值列表；double类型

      - 存在7个元素，为机器人的7个关节

      - 元素存在4个元素，第1位：主轴，2-4：从动轴

    - "slave_target":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人主轴及从动轴编码器位置目标值列表；double类型

      - 存在7个元素，为机器人的7个关节

      - 元素存在4个元素，第1位：主轴，2-4：从动轴

    - "slave_reality_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人外部轴主轴及从动轴编码器位置实际值列表；double类型

      - 存在5个元素，为机器人的5个外部轴

      - 元素存在4个元素，第1位：主轴，2-4：从动轴

    - "slave_target_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人外部轴主轴及从动轴编码器位置目标值列表；double类型

      - 存在5个元素，为机器人的5个外部轴

      - 元素存在4个元素，第1位：主轴，2-4：从动轴

{

"reality":[0,0,0,0,0,0,0],

"realitySync":[0,0,0,0,0],

"robot":1,

"target":[0,0,0,0,0,0,0],

"targetSync":[0,0,0,0,0],

"slave_reality":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_target":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_reality_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_target_sync":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

}

- 上位机查询机器人电机电流

  - 命令字：0x3268

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"robot":1

}

- 控制器回复上位机机器人电机电流

  - 命令字：0x3269

    - "current":[0,0,0,0,0,0,0],

      - 机器人电机电流列表；double类型，单位‰

      - 长度为7

    - "max_current":[0,0,0,0,0,0,0],

      - 机器人最大电机电流列表；double类型，单位‰

      - 长度为7

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

    - "sync_current":[0,0,0,0,0],

      - 机器人外部轴电机电流列表；double类型，单位‰

      - 长度为5

    - "sync_max_current":[0,0,0,0,0]

      - 机器人外部轴最大电机电流列表；double类型，单位‰

      - 长度为5

    - "slave_current":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人主轴及从动轴电机最大电流列表；double类型，单位‰

      - 存在7个元素，为机器人的7个关节

      - 元素存在4个元素，第1位：主轴，2-4：从动轴

    - "slave_max_current":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人外部轴主轴及从动轴电机电流列表；double类型，单位‰

      - 存在7个元素，为机器人的7个关节

      - 元素存在4个元素，第1位：主轴，2-4：从动轴

    - "slave_sync_max_current":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人外部轴主轴及从动轴电机最大电流列表；double类型，单位‰

      - 存在5个元素，为机器人的5个外部轴

      - 元素存在4个元素，第1位：主轴，2-4：从动轴

{

"current":[0,0,0,0,0,0,0],

"max_current":[0,0,0,0,0,0,0],

"robot":1,

"sync_current":[0,0,0,0,0],

"sync_max_current":[0,0,0,0,0],

"slave_current":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_max_current":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_sync_max_current":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

}

- 上位机查询机器人电机温度

  - 命令字：0x326A

    - "robot":1

      - 当前机器人号；int类型，范围[1,4]

{

"robot":1

}

- 控制器回复上位机机器人电机温度

  - 命令字：0x326B

    - "temperature":[0.0,0.0,0.0,0.0,0.0,0.0,0.0]

      - 机器人电机温度列表；double类型，单位°/C

      - 长度为7

    - "maxTemperature":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

      - 机器人最大电机温度列表；double类型，单位°/C

      - 长度为5

    - "temperatureSync":[0.0,0.0,0.0,0.0,0.0],

      - 机器人外部轴电机温度列表；double类型，单位°/C

      - 长度为7

    - "maxtemperatureSync":[0.0,0.0,0.0,0.0,0.0]

      - 机器人外部轴最大电机温度列表；double类型，单位°/C

      - 长度为5

    - "slave_temperature":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人主轴及从动轴伺服温度列表；double类型，单位°/C

      - 存在7个元素，为机器人的7个关节

      - 元素存在4个元素，第1位：主轴，2-4：从动轴

    - "slave_temperature_max":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人主轴及从动轴伺服最大温度列表；double类型，单位°/C

      - 存在7个元素，为机器人的7个关节

      - 元素存在4个元素，第1位：主轴，2-4：从动轴

    - "slave_temperature_sync_out":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人外部轴主轴及从动轴伺服温度列表；double类型，单位°/C

      - 存在5个元素，为机器人的5个外部轴

      - 元素存在4个元素，第1位：主轴，2-4：从动轴

    - "slave_temperature_sync_out_max":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

      - 机器人外部轴主轴及从动轴伺服最大温度列表；double类型，单位°/C

      - 存在5个元素，为机器人的5个外部轴

      - 元素存在4个元素，第1位：主轴，2-4：从动轴

{

"temperature":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

"maxTemperature":[0.0,0.0,0.0,0.0,0.0,0.0,0.0],

"temperatureSync":[0.0,0.0,0.0,0.0,0.0],

"maxtemperatureSync":[0.0,0.0,0.0,0.0,0.0],

"slave_temperature":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_temperature_max":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_temperature_sync_out":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]],

"slave_temperature_sync_out_max":[[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0],[0,0,0,0]]

}